INTRODUCTION TO DIGITAL SYSTEMS, SAMPLING AND ENCODING

Electronic devices becoming digital.

What Does “Digital” Mean?

Digital signal:

- Finite possible values

  - Ex: button pressed on a keypad

Analog signal:

- Infinite possible values

  - Ex: voltage on a wire created by

microphone

Binary digital signal  only 2 possible values ( 0 and 1 ). We will only consider binary digital signals.

1 binary digit is a bit.

Binary is popular bc transistors (the basic digital electric component) operate using 2 voltages and storing/transmitting one of 2 values is easier than 3 or more (e.g., loud beep or quiet beep, reflection or no reflection).

Digitalization benefit:

- Analog signal (e.g., audio, video) may lose quality

  - Voltage levels not saved/copied/transmitted perfectly

- Digitized version enables near-perfect save/cpy/tran.

  - “Sample” voltage at particular rate, save sample using bit encoding

  - Voltage levels still not kept perfectly

  - We can distinguish 0s from 1s

Neden tekrar analog sinyale dönüştürüyoruz?

Garip sesler çıkar. Kulaklarımız analog şekilde duyuyor.

Digitized Audio: Compression Benefit

Digitized audio can be compressed.

- e.g., MP3

- A CD can hold about 20 songs uncompressed, but about 200 compressed

Compression also done on digitized pictures (jpeg), movies (mpeg), and more.

Example compression scheme:

- 00 means 0000000000

- 01 means 1111111111

- 1X means X

0000000000 0000000000 0000001111 1111111111     00 00 10000001111 01

Digitization has many other benefits too.

How Do We Encode Data as Binary for Our Digital System?

Some inputs inherently binary

- Button: not pressed ( 0 ) , pressed ( 1 )

Some inputs inherently digital

- Just need encoding in binary

- e.g., multi-button input: encode red=001, blue=010, …

Some inputs analog

- Need a2d conversion

- As done in earlier, sample and encode with bits

How to Encode Text: ASCII, Unicode?

ASCII: 7 (or 8) bit encoding of each letter, number, or symbol

Unicode: Increasingly popular 16 bit encoding. Encodes characters from various world languages.

2

10

110     =  6

Decimal to Hex    convert to binary first, then binary to hex

With microprocessors so easy, cheap, and available, why design a digital circuit?

- Microprocessor may be too slow

- Or too big, power hungry, or costly

Digital systems surround us

- Inside computers

- Inside many other electronical devices (embedded systems)

Digital systems use 0s and 1s

- Encoding analog signals to digital can provide many benefits

  - e.g., audio -- higher quality storage/transmission, compression, etc.

- Encoding integers as 0s and 1s: Binary numbers

Microprocessors (themselves digital) can implement many digital systems easily and inexpensively

- But often not good enough -- need custom digital circuits