DATAPATH COMPONENT

Datapath component (RTL component : Register Transfer Level) is a circuit which stores/process data.

Datapath: Circuits has datapath components.

Register is a datapath component.

Register with Parallel Load

Add 2x1 MUX to front of each flip-flop

Register’s load input selects MUX input to pass

- load = 0    existing flip-flip value

- load = 1    new input value

When load = 0, I am storing the data.
When load = 1, I am taking new data from input lines.

Remember: at each clock cycle, we have to take an action

Register Example Using the Load Input: Weight Sampler

Buses

N-bit bus: N wires to carry N-bit data item

- Circuit drawings can become cluttered

Convention for drawing buses:

- Single bold line and/or small angled line across

Register Example: Above-Mirror Display

T : temperature
A : average fuel consumption
I : instantaneous fuel consumption
M : miles remaining

C : data that is one of these 4

We should activate the correct register for the type of the C.

You can set x and y to 01 to see 0001010 at the output D.

Register Example: Computerized Checkboard

We use decoder to activate the proper register. To indicate which register stores the data.

Data is going to be feed to all registers but only one of them will be active.

Shift Register (Kayıkçı)

1101 -------shift right-------> 0110     (rightmost bit is dropped, assume 0 shifted into leftmost bit)

0100  0010  0001  0000

4 bit right shift register:

shr : shift right
shr_in : shift right input

e, s1, s0, c come from car computer. User controls which data should be shown in the mirror  x, y

I am sending 1 bit for each clock signal.

If we set s1s0 to 01, A will be active. We are saying that “For 8 different bits (cycles), I will set data to A.”. After 8 clock cycle, my data is gonna be stored in register 1 (A).

For example our data is 00010000 for average consumption (A). We will set s1s0 to 01 bc we want to activate register 1.

At first clock signal reg1’s output will be:

- 0 _ _ _   _ _ _ _

At second clock signal:

- 0 0 _ _    _ _ _ _

At the end:

- 0001 0000

8 bit boyunca s1s0, 01 olarak kalacak. Eğer 8 bit sonunda bir şey gönderilmiyorsa e (enable) 0 yapılır. If e is 0, all of the outputs of decoder are 0. If e = 0, registers are not activated. Registers store the same data.

At 8 clock cycles, I will shift the data step by step because data cable (black c) is 1 wire cable. There is no parallel data. So there is going to be 1 data at each time in the cable.

Multitask Register

Tasks:

- Keep the current state (maintain)

- Parallel load

- Shift right

- Shift left

We use 4x1 MUX:

| s1 | s0 | operation |
| --- | --- | --- |
| 0 | 0 | maintain |
| 0 | 1 | load |
| 1 | 0 | shift right |
| 1 | 1 | shift left |

Üstteki s3, s2, s1, s0  I3 , I2, I1, I0 olacak.

We can have separate input for each task if we want:

- ld : load

- shr : shift right

- shl : shift left

If none of them is chosen, maintain state will be active.

| ld | shr | shl | operation | s1 | s0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | maintain | 0 | 0 |
| 0 | 0 | 1 | shift left | 1 | 1 |
| 0 | 1 | 0 | shift right | 1 | 0 |
| 0 | 1 | 1 | shift right | 1 | 0 |
| 1 | 0 | 0 | load | 0 | 1 |
| 1 | 0 | 1 | load | 0 | 1 |
| 1 | 1 | 0 | load | 0 | 1 |
| 1 | 1 | 1 | load | 0 | 1 |

shift right has priority over shift left.

load has priority over shit right and shift left.

s0 : ld + ld’shr’shl

s1 : ld’shr + ld’shr’shl = ld’(shr + shr’shl) = ld’(shr + shl) = ld’shr + ld’shl

Design Registers

- STEP 1: Define the MUX size

- STEP 2: Create MUX operation table

- STEP 3: Connect MUX inputs

- STEP 4: Map control lines

EXAMPLE: Create a register (4 bits) for load, shift left, clear, and set operations using the control inputs (ld, shl, clr, set):

clr will load 0
set will load 1

We have 5 operations, 5th one is maintain. Maintain is default.

Define the MUX size: We need 8x1 MUX because we have 5 tasks.

Create MUX table

| s2 | s1 | s0 | operation | inputs of the MUX |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | maintain | 0 |
| 0 | 0 | 1 | load | 1 |
| 0 | 1 | 0 | shift left | 2 |
| 0 | 1 | 1 | clear | 3 |
| 1 | 0 | 0 | set | 4 |
| 1 | 0 | 1 | maintain | 5 |
| 1 | 1 | 0 | maintain | 6 |
| 1 | 1 | 1 | maintain | 7 |

Connect MUX inputs

Q-1    Sonraki registerın outputu, sonraki register yoksa  shl_in

Map control lines

| ld | shl | set | clr | s2 | s1 | s0 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | … | … | … |
| 0 | 1 | 0 | 1 |  |  |  |
| 0 | 1 | 1 | 0 |  |  |  |
| 0 | 1 | 1 | 1 |  |  |  |
| 1 | 0 | 0 | 0 |  |  |  |
| 1 | 0 | 0 | 1 |  |  |  |
| 1 | 0 | 1 | 0 |  |  |  |
| 1 | 0 | 1 | 1 |  |  |  |
| 1 | 1 | 0 | 0 |  |  |  |
| 1 | 1 | 0 | 1 |  |  |  |
| 1 | 1 | 1 | 0 |  |  |  |
| 1 | 1 | 1 | 1 |  |  |  |

You have to define priority for some cases. At the end you get equation for each s2, s1, and s0.

We have created a datapath component.

OTHER DATAPATH COMPONENTS

- Adders

- Subtractor

- Comparator

- Incrementer

- ALU

Son slayt, sayfa 21’den başlıyor.