Register Files

Accessing one of several registers is:

- OK if just a few registers

- Problematic when many

- Ex: Earlier above-mirror display, with 16 registers

  - Much fanout (branching of wire): Weakens signal

  - Many wires: Congestion

Data (9) is decreasing step by step.

MxN register file: Efficient design for one-at-a-time write/read of many registers (M register, N bit data)

- Consider 16x32-bit registers (16 registers, bit size: 32)

You can use these registers for different datas. One part can hold a data, other part can hold another data.

Internal design uses drivers and bus

three-state driver has the same purpose with driver  strength the data

In three-state driver, if c is 0, it behaves like open circuit

Register-File Example: Above-Mirror Display

16 32-bit registers that can be written by car’s computer, and displayed

- Use 16x32 register file

- Simple, elegant design

Register file hides complexity internally

- because only one register needs to be written and/or read at a time, internal design is simple

NEW design

Example: Find the greatest common divider of A and B

//Inputs:
	byte a, b;
	bit go;

//Outputs
	byte gcd;
	bit done;

while (1){

while(!go);

done = 0;

while (a != b){

if (a > b) a = a - b;

else b = b - a;

}

gcd = a;

done = 1;

}

local registers: regA and regB

Design the datapath:

- 2 local registers to hold the data:

- regA

- regB

- Comparator

- 2 substractors

Sadece go 1’ken (döngünün başında) new_data_ld 1 olur ki yeni gelen veriler yüklensin.