Time Domain Analysis: LTIC Systems

LTIC  Continuous-time systems

LTID  Discrete-time systems

LTI: Linear time invariant

Introduction

Input f(t) and output y(t) of an LTIC system is related by linear differential equations,

Q(D) . y(t) = P(D) . f(t)       Linear System Equation

Theoratically, m and n can be any value but practically m <= n due to noise considerations.

Response of a System

Total response = Zero-Input Response – Zero-State Response

Zero-Input Response

- System response when f(t) = 0

- It is the result of internal system conditions (energy stores) alone

- Independent of external input f(t)

Zero-State Response

- System response when all the initial conditions are 0 (zero)

- You just look at the effect of the input signal on the response

The Zero-Input Response

The zero-input response y0(t) is the solution of

Q(D) . y(t) = P(D) . f(t)

when f(t) = 0. That is,

Q(D) . y(t) = P(D) . 0 = 0

Let’s consider the simplest case of 1st order system (when n = 1),

- (D + a0) . y0(t) = 0

- D . y0(t) + a0 . y0(t) = 0

- D . y0(t) = - a0 . y0(t)

- ⅆⅆt y0(t) = - a0 . y0(t)	------> How can we solve this equation to determine y0(t)?

2 main approaches to solve the equation:

- Systematic way

- Heuristic reasoning

  - Only exponential functions have such property, where the derivative has the same form of itself. Let’s assume that the solution is:

    - y0(t) = c . ⅇλt

    - ⅆⅆt y0(t) = c . λ . ⅇλt = - a0 . y0(t)

    - λ = - a0

    - Thus the solution is ---> y0(t) = c . ⅇ-a0 .  t  (c is arbitrary constant)

To generalize this solution, consider the following equation,

- Q(D) . y0(t) = 0

- When roots are non-repeated:

  - General solution is:

(Assumes λi are distinct)

  - where c1, c2, …, cn are arbitrary constants determined by constraints on the solution.

  - ---------------------------------------------------------------------------------------------------------------------

  - Q(λ) = Q(D) |D = λ

    - Q(λ) = (λ – λ1) . (λ – λ2) . … . (λ – λn)     --> characteristic polynomial

    - λ1, λ2, …, λn  characteristic roots

      - parameters or the constants that shows the behaviour of your system

  - ⅇλi .  t (i = 1, 2, …, n)  characteristic modes

  - ---------------------------------------------------------------------------------------------------------------------

  - There is a characteristic mode for each characteristic root of the system.

  - Therefore, the zero-input response (y0(t)) is a linear combination of the characteristic modes of the system.

- When roots are repeated:

  - (D – λ)2 . y0(t) = 0

  - square means system has 2 roots and they are repeated

  - The solution is:

    - y0(t) = (c1 + c2 . t) . ⅇλt

  - General case:

    - (D – λ)r . y0(t) = 0

  - The characteristic modes are:

    - ⅇλt,   t . ⅇλt,   t2 . ⅇλt, … , tr-1 . ⅇλt

  - The solution is:

Example:

- For the given LTIC system,

  - (D2 + 6D + 9) . y(t) = (3D + 5) . f(t)

- Determine y0(t) if y0(0) = 3, y0’(0) = -7  ------> Initial conditions

- --------------------------------------------------------------------------------------------------------------------------------

- First find characteristic polynomial by replacing D with λ

  - Q(λ) = Q(D) |D = λ  =  (D2 + 6D + 9)|D = λ

  - Q(λ) = λ 2 + 6 λ + 9 ------> CHARACTERISTIC POLYNOMIAL

    - (λ + 3) . (λ + 3)

    - λ1 = -3,    λ2 = -3   ---> REPEATED ROOTS

    - Therefore the characteristic modes are ⅇλ1 .  t and ⅇλ2 .  t

      - ⅇ-3 .  t and ⅇ-3 .  t

- The general solution:

  - y0(t) = (c1 + t . c2) . ⅇλt

  - y0(t) = (c1 + t . c2) . ⅇ-3t

- To find the unique solution  determine c1 and c2

  - y0(0) = 3  c1 = 3

  - y0’(t) = [-3 . (c1 + c2 . t) + c2] . e-3t

  - y0’(t=0) = -3c1 + c2 = -7  c2 = 2

- Therefore, zero-input response is

  - y0(t) = (3 + 2t) . ⅇ-3t

Unit Impulse Function

The Unit Impulse Response (h(t))

δ(t) is used to determine the response of a linear system.

push this h(t) to output

if f(t) = δ(t)  y(t) = h(t)

for an LTIC system,

- Q(D) . y(t) = P(D) . f(t)

h(t) is given by,

- h(t) = bn . δ(t) + [P(D) . y0(t)] . u(t)     ----> u(t) is unit step function

- where bn is the coefficient of nth order in Q(D) and initial conditions are:

  - y0n-1(0) = 1,

  - y0(0) = y0’(0) = y0’’(0) = … = y0n-2(0) = 0

Example:

- Determine h(t) of the following system,

  - (D2 + 3D + 2) . y(t) = D . f(t)

- --------------------------------------------------------------------------------------------------------------------------------

- h(t) = bn . δ(t) + [P(D) . y0(t)] . u(t)     ---> we don’t know bn and y0(t)

- P(D) = D

- n = 2

- We need to find b2 now.

  - b2 . D2 + b1 . D1 = P(D) = D   ---> b2 = 0,  b1 = 1

  - bm . Dm  +  bm-1 . Dm-1 + …

- b2 = 0, y0(t) = ?

  - Q(λ) = Q(D) |D = λ  =  (D2 + 3D + 2)|D = λ

  - Q(λ) = λ 2 + 3λ + 2 = (λ + 1) . (λ + 2)

  - λ1 = -1    ≠   λ2 = -2      -----> They are non-repeated

  - y0(t) = c1 . ⅇλ1 .  t + c2 . ⅇλ2 .  t + … + cn . ⅇλn .  t

  - We have 2 roots so:

    - y0(t) = c1 . ⅇλ1 .  t + c2 . ⅇλ2 .  t

    - y0(t) = c1 . ⅇ-t + c2 . ⅇ-2t

- Now we need to find c1 and c2.

- By definition:

  - y0n-1(0) = 1       y0(0) = 0       n=2

  - y0’(0) = 1

- y0(t) = c1 . ⅇ-t + c2 . ⅇ-2t

- y0(t=0) = c1 + c2 = 0  c1 = -c2

- y0’(t) = -c1 . e-t – 2 . c2 . e-2t |t=0 = 1

- - c1 - 2c2 = 1

- c1 = 1    and    c2 = -1

- y0(t) = ⅇ-t - ⅇ-2t

- h(t) = 0 . δ(t) + [D . (ⅇ-t - ⅇ-2t)] . u(t)

- h(t) = (-ⅇ-t + 2ⅇ-2t) . u(t)

The Zero-State Response

y(t) = -∞∞fz .  ht-zdz

Convolution (integral)

z is actually tau

Some Important Properties of Convolution

The commutative:

- f1(t) * f2(t) = f2(t) * f1(t)

- * is convolution operation

The distributive:

- f1(t) * [f2(t) + f3(t)] = f1(t) * f2(t) + f1(t) * f3(t)

The associative:

- f1(t) * [f2(t) * f3(t)] = [f1(t) * f2(t)] * f3(t)

The shift:

- f1(t) * f2(t) = c(t)

- f1(t) * f2(t – T) = c(t – T)

- f1(t – T) * f2(t) = c(t – T)

Convolution with impulse function:

- f(t) * δ(t) = f(t)

The width:

- f1(t)  T1 duration

- f2(t)  T2 duration

- f1(t) * f2(t) = c(t)  T1 + T2 duration

There are 2 ways to perform convolution:

- By equation

- Visually

Example:

- Consider an LTIC system with h(t) = e-2t . u(t)

- Determine y(t) (zero-state response) for input f(t) = e-t . u(t)

- --------------------------------------------------------------------------------------------------------------------------------

- --------------------------------------------------------------------------------------------------------------------------------

Unit step function

- First way is solve with equation:

- y(t) = -∞∞fz .  ht-zdz

  - f(z) = f(t)|t=z = e-z . u(z)

  - h(t-z) = h(t)|t=t-z = e-2(t-z) . u(z)

  - y(t) = 0te-ze-2(t-z)dz = e-2t0tezdz = e-2t . ez|0t

  - y(t) = e-2t . (et – 1),    t >= 0

  - y(t) = 0,   t < 0, then:

  - y(t) = (e-t – e-2t) . u(t)   ----> zero state response

Example (Graphical understanding of convolution):

- x1(t) is my input signal

- x2(t) is my system impulse response

- Now we feed x1(t) with system x2(t)

- blue  input signal                                                                                 red  system impulse response

- - infinity to 0 is 0. from t to infinity is also 0.

- Only non-zero region is overlapped region.

t-1

- After t >= 2, there is no overlap so we slided over the system. After t >= 2, we obtain the output.

System Stability

If an LTIC system has n distinct characteristic roots λ1, λ2, …, λn, then the zero input response is:

0	, Real(λ) < 0     ------> Left hand of complex plane (LHP) – for stable systems

∞	, Real(λ) > 0     ------> Right hand of complex plane (RHP)

limt→∞ⅇλt =

marginally stable means on top of y axis.

(Asymptotically) Stable: All λi are in LHP regardless of repeated and non-repeated.

Unstable: Either one or both of the following conditions;

- at least one root is in the RHP

- there are repeated roots on the imaginary axis

Marginally Stable: There are no roots in the RHP and there are some non-repeated roots on the imaginary axis.

Example:

- --------------------------------------------------------------------------------------------------------------------------------

- The characteristic polynomials are:

- The characteristic roots are:

- Put them in imaginary and real plane: